Chapter 3

# Testing

Three sets of tests are presented in this chapter to demonstrate the performance of the centre-surround feature contrast detection algorithm.  The first two tests measure the ability of the algorithm to detect and accurately quantify orientation contrast and intensity contrast.  The final test measures the effect of separation of image elements on the feature contrast measurement.

All test images are 256 x 256 pixels, and consist of a black border surrounding an array of horizontal line segments (distracters) with a target line segment, differing from the distracters in either orientation or intensity, located at a random position within the array.  Each line segment is either white or grey on a black background, and each is contained in a 16 x 16 pixel block (Figure 1).  In this chapter, a block containing a single line segment is referred to as a line element. In the multiple-resolution representation, two levels of reduction are used, so the resulting contrast map is 64 x 64 pixels with each line element represented by a 4 x 4 pixel square.

| Figure 1: Line elements: |  |  |  |
| --- | --- | --- | --- |
| (a) | (b) | (c) | (d) |

(a) Orientation = 0, Intensity = 255 (b) Orientation = 45, Intensity = 255 (c) Orientation = 90, Intensity = 255 (d) Orientation = 0, Intensity = 130

The level of contrast for an image is quantified as the “contrast ratio”.  This is calculated by first averaging the 16 pixels representing each line element in the final contrast map, so that each element is represented by a single contrast value.  The contrast values for all distracter elements, except those affected by the black border, are then averaged, and the contrast value of the target element is divided by the average of the distracters to achieve the final contrast ratio.

This contrast ratio is specifically designed for the images presented in this paper.  Elements are always contained within a block of 16 pixels, never across multiple blocks, and the calculations take advantage of this fact.   The contrast ratio would be affected in two ways if this assumption were false.  The first is if the elements were of the 16-pixel scale, but the borders of the elements did not line up with the borders used in calculating the contrast ratio.  That is, the element spanned several (up to four) blocks.  In this case, the contrast ratio value would be decreased, but in most cases (except those with very low levels of contrast) the value would still be useful for comparison.  The second possible way the effectiveness of the contrast ratio could be affected is if the scale of the elements was considerably larger than 16 pixels.  As the element size increases, the number of blocks reporting a high level of contrast will increase, decreasing the result returned by the contrast detector until the value becomes meaningless.

## Test 1 - Orientation Contrast

To test the sensitivity of the algorithm to orientation contrast, the target element is a line segment at an angle  from the horizontal, where  varies from 0 to 180 by increments of 10 (Figure 2). The sum of pixel intensities for all line elements is equal, regardless of the line orientation.

The dimensions of the centre Gaussian, 5 x 5, allow it to completely cover only a single element, and the dimensions of the surround Gaussian, 13 x 13, allow it to cover a 3 x 3 block of elements, the smallest block to completely surround the centre element.  The current implementation requires the Gaussian size to be set as a parameter of the system.  An implementation involving multiple scales is discussed in the conclusion under future work.

The results for these tests are shown in Table 1 and Figure 3.  No contrast in the image is indicated by a ratio of one, as can be seen when the target is at orientations 0 and 180, equal to the angle of the distracters.  From 0, the function increases monotonically to the maximum contrast, which occurs when the target element is at an angle of 90, orthogonal to the distracters, and then monotonically decreases again to 1 at 180.

| Figure 2: Sample input images for orientation contrast testing and the resulting contrast maps. Orientation of the background elements is 0, and the orientation of the target element varies. |  |  |  |
| --- | --- | --- | --- |
| (a) Target Orientation = 20 |  | (b) Target Orientation = 60 |  |
| (c) Target Orientation = 100 |  | (d) Target Orientation = 160 |  |

These results confirm that the algorithm is able to detect orientation contrast and to accurately quantify the relative contrast within the image.  The final contrast maps are presented beside the images in Figure 2.  Visual inspection of these maps verifies that a greater difference between the angle of the target and the angle of the distracters results in a greater contrast measurement by the algorithm.

|  | Orientation | Contrast Ratio |  | Orientation | Contrast Ratio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  | 100 | 16.9629 |  |
|  | 10 | 3.40609 |  | 110 | 15.5295 |  |
|  | 20 | 5.43968 |  | 120 | 13.5974 |  |
|  | 30 | 8.02259 |  | 130 | 11.4371 |  |
|  | 40 | 9.80037 |  | 140 | 9.80037 |  |
|  | 50 | 11.8092 |  | 150 | 7.79023 |  |
|  | 60 | 14.0904 |  | 160 | 5.43908 |  |
|  | 70 | 15.4592 |  | 170 | 3.40655 |  |
|  | 80 | 17.7942 |  | 180 | 1 |  |
|  | 90 | 19.4716 |  |  |  |  |
| Table 1: Contrast ratio obtained for images with horizontal distractors and varying target orientation |  |  |  |  |  |  |

Figure 3: Contrast ratio obtained for images with horizontal distractors and varying target orientation

## Test 2 - Intensity Contrast

To test the sensitivity of the algorithm to intensity contrast, the images for this test contain horizontal distracters in which the intensity of each pixel is set to 130 out of 255.  The intensity of the horizontal target element varies from 0 to 250 by increments of 10 (Figure 4). The results for the intensity tests are shown in Figure 4, Figure 5 and Table 2.  The no-contrast case for these tests occur when the intensity of the target is 130, equal to the intensity of the distracter elements.  As in the orientation contrast test, the contrast ratio is one for the no-contrast case and increases in both directions as the difference in intensity, and with it the amount of contrast, increases.  The final contrast maps are shown beside input images in Figure 4.  Visual inspection of these maps confirms the quantitative analysis that the measure of contrast detected by the algorithm increases as the difference in intensity between the target and distracter elements increases.  These results verify that the algorithm is able to successfully identify and measure relative intensity contrast within an image.

| Figure 4: Sample input images for intensity contrast testing and the resulting contrast maps. Background elements have intensity of 130/255, and the intensity of the target element varies. |  |  |  |
| --- | --- | --- | --- |
| (a) Target Intensity = 30/255 |  | (b) Target Intensity = 90/255 |  |
| (c) Target Intensity = 140/255 |  | (d) Target Intensity = 210/255 |  |

|  |  | Intensity | Contrast Ratio |  | Intensity | Contrast Ratio |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 10.4757 |  | 130 | 1 |  |  |
|  |  | 10 | 9.83603 |  | 140 | 1.09671 |  |  |
|  |  | 20 | 8.92434 |  | 150 | 1.56683 |  |  |
|  |  | 30 | 8.40554 |  | 160 | 2.52943 |  |  |
|  |  | 40 | 7.49294 |  | 170 | 3.3204 |  |  |
|  |  | 50 | 6.66838 |  | 180 | 4.08876 |  |  |
|  |  | 60 | 5.8536 |  | 190 | 4.94911 |  |  |
|  |  | 70 | 5.04116 |  | 200 | 5.67333 |  |  |
|  |  | 80 | 4.23985 |  | 210 | 6.4726 |  |  |
|  |  | 90 | 3.38156 |  | 220 | 7.27876 |  |  |
|  |  | 100 | 2.52883 |  | 230 | 8.0265 |  |  |
|  |  | 110 | 1.69082 |  | 240 | 8.86904 |  |  |
|  |  | 120 | 1.12987 |  | 250 | 9.89877 |  |  |
|  | Table 2: Contrast ratio obtained for an image with the intensity of distractor elements set to 130/255 and varying target intensity. The “Intensity” value refers to the intensity of the target element. |  |  |  |  |  |  |  |
| Figure 5: Contrast ratio obtained for an image with the intensity of distractor elements set to 130/255 and varying target intensity. The “Intensity” value refers to the intensity of the target element. |  |  |  |  |  |  |  |  |

## Test 3 - Separation

This series of tests was designed to determine the effect of an increase in the distance between elements.  Because the centre-surround feature contrast detection algorithm measures local contrast, it is expected that an increase in separation of elements will result in a decreased contrast measurement.  The target element for all trials in this series is a vertical line with intensity equal to that of the horizontal distracters.  Five trials were run with separation of two pixels between line segments on the first trial, and separation increasing by four pixels in each subsequent trial (Figure 6). The additional pixels between elements are not included in the calculation of contrast ratio.  Each element is considered to be a 16 x 16 pixel square in the original image, or 4 x 4 in the final contrast map.  As the separation between elements increases, the number of elements decreases, but the size of the elements remains the same. The results for the separation trials are shown in Figure 6, Figure 7 and Table 3.  As can be seen in the graph, table and contrast maps, the contrast ratio decreases as the amount of separation increases.  This confirms that the algorithm responds appropriately to decreased proximity causing decreased local contrast by a decreased contrast measurement.

| Figure 6: Sample input images for separation testing and the resulting contrast maps. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (a) Separation = 6 pixels |  |  | (b) Separation = 14 pixels |  |  |  |
| (c) Separation = 18 pixels |  |  |  |  |  |  |

|  | Separation | 2 | 6 | 10 | 14 | 18 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Contrast Ratio | 18.948 | 9.47941 | 2.8951 | 1.47925 | 1.06746 |  |
| Table 3: Contrast ratio obtained for an image with 90 orientation ratio when the elements are moved apart from each other. The “Separation” value refers to the number of blank pixels between elements. |  |  |  |  |  |  |  |

Figure 7: Contrast ratio obtained for an image with 90 orientation ratio when the elements are moved apart from each other. The “Separation” value refers to the number of blank pixels between elements.

## Test 4 - Noise

The section contains two sets of tests: one testing the effect of noise in the image on the detection of orientation contrast, and the other testing the effect of noise in the image on the detection of intensity contrast.  It is expected that with increased noise, the ability to detect contrast will steadily decrease to the point where contrast can no longer be reliably detected.

### Effect of Noise on Detection of Orientation Contrast

All images in this set of tests were formed by adding varying amounts of noise to the no-noise image shown in Figure 8, and therefore all images have orientation contrast of 90.  Noise was added according to a Gaussian distribution with increasing values of  to increase the amount of noise.  Figure 8 shows images that are used in this test and illustrates the effect of increasing the value of .

| Figure 8: Sample input images for testing the effects of noise on orientation contrast and the resulting contrasting maps. The target element has orientation contrast of 90 and the amount of noise added varies. |  |  |  |
| --- | --- | --- | --- |
| (a) Standard Deviation of Noise Added = 0 |  | (b) Standard Deviation of Noise Added = 30 |  |
| (c) Standard Deviation of Noise Added = 60 |  | (d) Standard Deviation of Noise Added = 100 |  |

The results of these tests are shown in Figure 8, Figure 9 and Table 4.  In general, the ability of the algorithm to accurately detect orientation contrast decreases as noise is added to the image.  For  < 50, the noise has very little effect on the contrast measurement.  Measured contrast decreases monotonically for  = 50 to  = 120, after which point the contrast ratio hovers around 1 – the “no-contrast” indicator.  At this point the noise has interfered with the measurement sufficiently to render the contrast measurement useless.

Because orientation contrast is based solely upon the output of the edge operator, the effect of noise on the algorithm is due to the sensitivity of the edge operator to noise.  An edge operator with a greater tolerance for noise would require a larger value of  to reach the point where the contrast measurement is useless.

|  | Standard Deviation of Noise Added | Contrast Ratio |  | Standard Deviation of Noise Added | Contrast Ratio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 19.4852 |  | 100 | 3.76896 |  |
|  | 10 | 19.2517 |  | 110 | 3.33059 |  |
|  | 20 | 19.5017 |  | 120 | 2.43007 |  |
|  | 30 | 18.282 |  | 130 | 1.49821 |  |
|  | 40 | 17.3708 |  | 140 | 0.836841 |  |
|  | 50 | 15.0213 |  | 150 | 1.81355 |  |
|  | 60 | 12.2592 |  | 160 | 1.88734 |  |
|  | 70 | 9.83904 |  | 170 | 0.826554 |  |
|  | 80 | 7.51405 |  | 180 | 1.09178 |  |
|  | 90 | 5.95041 |  | 190 | 1.46978 |  |
| Table 4: Contrast ratio obtained for an image with 90 orientation ratio when noise is added. The “Noise Added” value refers to the  of the Gaussian used to determine the noise distribution. |  |  |  |  |  |  |

Figure 9:  Contrast ratio obtained for an image with 120/255 intensity contrast when noise is added.  The “Noise Added” value refers to the  of the Gaussian used to determine the noise distribution.

### Effect of Noise on Detection of Intensity Contrast

The images for this set of tests is based on the “no-noise” image shown in Figure 10.  Distractor elements in this image have an intensity of 130/255 and the target element has an intensity of 250/255.  Noise is added to this image by the same procedure used to add noise to the orientation contrast image in the previous set of tests, that is, by a Gaussian distribution with varying values of .

| Figure 10: Sample input images for testing the effects of noise on intensity contrast and the resulting contrasting maps. Background elements have intensity of 130/255, the intensity of the target element is 250/255, and the amount of noise added varies. |  |  |  |
| --- | --- | --- | --- |
| (a) Standard Deviation of Noise Added = 0 |  | (b) Standard Deviation of Noise Added = 20 |  |
| (c) Standard Deviation of Noise Added = 40 |  | (d) Standard Deviation of Noise Added = 60 |  |

The results of this test are shown in Figure 10, Figure 11, and Table 5.  The pattern of these results is similar to the results for orientation contrast with noise in that the graph can be divided into three sections.  For low noise level, there seems to be very little effect on the contrast measurement.  Following this, the central part of the graph shows a monotonic decrease in contrast ratio with increasing noise.  The final section shows the contrast ratio hovering around 1, the “no-contrast” value.

|  | Standard Deviation of Noise Added | Contrast Ratio |  | Standard Deviation of Noise Added | Contrast Ratio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 9.89877 |  | 100 | 1.18274 |  |
|  | 10 | 10.0401 |  | 110 | 1.14463 |  |
|  | 20 | 8.6881 |  | 120 | 1.55468 |  |
|  | 30 | 7.52518 |  | 130 | 2.31646 |  |
|  | 40 | 6.89435 |  | 140 | 1.05232 |  |
|  | 50 | 3.52135 |  | 150 | 0.716185 |  |
|  | 60 | 3.31347 |  | 160 | 0.723081 |  |
|  | 70 | 1.3233 |  | 170 | 0.639432 |  |
|  | 80 | 1.22506 |  | 180 | 0.89982 |  |
|  | 90 | 1.53364 |  | 190 | 1.25658 |  |
| Table 5: Contrast ratio obtained for an image with 120/255 intensity contrast when noise is added. The “Noise Added” value refers to the  of the Gaussian used to determine the noise distribution. |  |  |  |  |  |  |

Figure 11:  Contrast ratio obtained for an image with 120/255 intensity contrast when noise is added.  The “Noise Added” value refers to the  of the Gaussian used to determine the noise distribution.

The main difference between the intensity and orientation tests lies in the boundaries of the divisions of the three sections in the graph.  For orientation, the drop in contrast ratio goes from  = 50 to  = 120, while for intensity the drop is much sharper, from  = 50 to  = 70.  The increase in noise has a direct effect on the intensity contrast measurement whereas with orientation contrast, the noise is somewhat filtered by the use of the edge operator, smoothing the results.  The intensity contrast measurement, without such a smoothing effect, drops much more sharply and reaches the point where no contrast is detected much more quickly.  A separate smoothing function applied to the image would reduce the effect of the noise and result in a graph more similar to the orientation graph.

## Test 5 – Background Intensity

To this point, all tests have been on images with a solid black background.  This set of tests looks at the effect of changing the intensity of the background.  Once again, there are two sets of tests; the first looks at orientation contrast and the second looks at intensity contrast.  It is expected that the intensity of the background will have little or no effect on the outcome of the contrast detection algorithm.

### Effect of Background Intensity on Detection of Orientation Contrast

These images all contain 90 orientation contrast and line elements with an intensity of 255/255.  The background intensity varies from 0/255 to 250/255.  Sample images are shown in Figure 12.

| Figure 12: Sample input images for testing the effects of background intensity on orientation contrast and the resulting contrasting maps. All images have 90 orientation contrast and all elements have intensity of 255/255. The intensity of the background varies. |  |  |  |
| --- | --- | --- | --- |
| (a) Background intenstity = 0/255 |  | (b) Background intensity = 70/255 |  |
| (c) Background intensity = 160/255 |  | (d) Background intensity = 250/255 |  |

It can be seen from the results shown in Figure 12, Figure 13, and Table 6 that the intensity of the background has absolutely no effect on the detection of orientation contrast.  This is because orientation contrast is based on the output of an edge operator.  Anywhere that no edge is detected is assigned a value of 0 on all edge maps, so the background pixels are assigned a value of 0 regardless of their intensity value.

|  | Background Intensity | Contrast Ratio |  | Background Intensity | Contrast Ratio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 19.4852 |  | 130 | 19.4852 |  |
|  | 10 | 19.4852 |  | 140 | 19.4852 |  |
|  | 20 | 19.4852 |  | 150 | 19.4852 |  |
|  | 30 | 19.4852 |  | 160 | 19.4852 |  |
|  | 40 | 19.4852 |  | 170 | 19.4852 |  |
|  | 50 | 19.4852 |  | 180 | 19.4852 |  |
|  | 60 | 19.4852 |  | 190 | 19.4852 |  |
|  | 70 | 19.4852 |  | 200 | 19.4852 |  |
|  | 80 | 19.4852 |  | 210 | 19.4852 |  |
|  | 90 | 19.4852 |  | 220 | 19.4852 |  |
|  | 100 | 19.4852 |  | 230 | 19.4852 |  |
|  | 110 | 19.4852 |  | 240 | 19.4852 |  |
|  | 120 | 19.4852 |  | 250 | 19.4852 |  |
| Table 6: Contrast ratio obtained for an image with 90 orientation contrast on a background of varying intensity. |  |  |  |  |  |  |

Figure 13:  Contrast ratio obtained for an image with 90 orientation contrast on a background of varying intensity.

### Effect of Background Intensity on Detection of Intensity Contrast

The images used to test the effect of background intensity on intensity contrast have a target element of 250/255 and distractor elements of 130/255.  As with the test on orientation contrast, the background intensity varies from 0/255 to 255/255.  Sample images are shown in Figure 14.

| Figure 14: Sample input images for testing the effects of background intensity on intensity contrast and the resulting contrasting maps. The target element has an intensity of 250/255 and background elements have an intensity of 130/255. The intensity of the background varies. |  |  |  |
| --- | --- | --- | --- |
| (a) Background intenstity = 0/255 |  | (b) Background intensity = 80/255 |  |
| (c) Background intensity = 160/255 |  | (d) Background intensity = 250/255 |  |

The results of these tests, shown in Figure 14, Figure 15, and Table 7, show that although the target element is obviously indicated in all contrast maps, the contrast ratio value is largely affected by the background intensity.  This is because the value of the background pixels is itself used in the calculation of intensity contrast.  In all cases, except when background intensity is 130/255, the contrast map is indicating contrast between line elements and the background in addition to indicating contrast among the line elements themselves.  Because of this, the overall ratio changes even though the target is the spot of highest contrast in all cases.

|  | Background Intensity | Contrast Ratio |  | Background Intensity | Contrast Ratio |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 9.89877 |  | 130 | 106.74 |  |
|  | 10 | 10.878 |  | 140 | 84.3789 |  |
|  | 20 | 11.4286 |  | 150 | 45.4307 |  |
|  | 30 | 12.3907 |  | 160 | 36.8369 |  |
|  | 40 | 13.7471 |  | 170 | 28.4859 |  |
|  | 50 | 15.4756 |  | 180 | 24.7069 |  |
|  | 60 | 17.7473 |  | 190 | 20.0431 |  |
|  | 70 | 19.8451 |  | 200 | 17.1781 |  |
|  | 80 | 23.0849 |  | 210 | 15.3671 |  |
|  | 90 | 29.6376 |  | 220 | 13.8525 |  |
|  | 100 | 37.8109 |  | 230 | 12.8214 |  |
|  | 110 | 46.2442 |  | 240 | 11.724 |  |
|  | 120 | 84.6098 |  | 250 | 10.7974 |  |
| Table 7: Contrast ratio obtained for an image with 120/255 intensity contrast on a background of varying intensity. |  |  |  |  |  |  |

Figure 15:  Contrast ratio obtained for an image with 120/255 intensity contrast on a background of varying intensity.

### Effect of Background Variation within an Image

Figure 16 shows an example of an image in which the background intensity varies from 0/255 to 255/255.  The horizontal bars have an intensity of 255/255, 130/255 and 0/255.  The effect of a change in background intensity is much more visible in a single image than in the multiple images of the previous tests because the contrast measurement is relative with the image.  In an image with a solid background, the intensity of the background makes little difference because the contrast relative to the background will be the same throughout the image.  In an image with a varying background, however, a single element can be in greater contrast with the background in one portion of the image than in another.

One interesting point seen in the intensity contrast results (Figure 16 b) results is that they highlight areas where the simultaneous contrast effect is seen in the original image.  The white line looks lighter against the dark portion of the background, the black line looks darker against the light portion of the background, and the grey line appears to transition from light to dark as the background changes from dark to light.  It is believed that low-level processing, and in particular a center-surround inhibition model is at least in part responsible for this type of illusion (Adelson 2000), and so it is not surprising that the results of the center-surround contrast detector align with the illusion.

| (a) Input Image | (b) Intensity Contrast | (c) Orientation Contrast |
| --- | --- | --- |

Figure 16:  An example of the effect of background variation within an image on intensity contrast.

## Summary

This chapter contains a description and the results of five sets of tests that illustrate the behaviour of the contrast detection algorithm.

Test 1 – Orientation  This test illustrates that the contrast detection algorithm is able to detect and accurately quantify orientation contrast.  The algroithm indicates no contrast when the target element is at the same orientation as the distractor elements.  As the orientation of the target element is changed (increased or decreased), the algorithm accurately reports increasing orientation contrast, reporting maximum contrast when the target is perpendicular to the distractors.

Test 2 – Intensity  This test illustrates that the contrast detection algorithm is able to detect and accurately quantify intensity contrast.  The algorithm indicates no contrast when the intensity of the target element is equal to the intensity of the distractor elements.  When the difference between the intensity of the target and the intensity of the distractors is increased, by either increasing or decreasing the intensity of the target element, the algorithm accurately reports an increase in intensity contrast.

Test 3 – Separation  This test illustrates that the contrast detection algorithm responds accurately to increased distance between adjacent line elements.  The algorithm indicates greatest contrast when the elements are placed in close proximity.  As the distance between elements is increased, the algorithm accurately reports ever decreasing contrast.

Test 4 – Noise  This test illustrates the effect of noise on the performance of the contrast detection algorithm.  The ability of the algorithm to accurately detect contrast decreases as noise is added to the image.  Noise has a greater effect on the detection of intensity contrast than on the detection of orientation contrast.

Test 5 – Background Intensity  This test illustrates the effect of background intensity on the performance of the contrast detection algorithm.  For images with a solid background, the intensity of the background has no effect on detection of orientation contrast and very little effect on the detection of intensity contrast.  In an image where the background varies, a greater difference between background and foreground results in a greater measure of both orientation and intensity contrast.